| Class or Subsidiary Risk |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Class 2 |  |  | Class 3 | Class 4 |  |  | Class 5 |  | Class 6 | Class 8 |
|  |  | Class 2 | Flammable Gas | Okay | 3 metres | Segregate | Segregate | Segregate | Segregate | Segregate | Segregate | Isolate | 3 metres | 3 metres |
|  |  |  | Non Toxic / Non Flammable Gas | 3 metres | Okay | Refer SDS | 3 metres | Refer SDS | Segregate | Refer SDS | Refer SDS | Segregate | Refer SDS | 3 metres |
|  |  |  | Oxidising Gas | Segregate | Refer SDS | Okay | Segregate | Segregate | Segregate | 3 metres | Refer SDS | Segregate | 3 metres | 3 metres |
|  |  | Class 3 | Flammable Liquids | Segregate | 3 metres | Segregate | Okay | 3 metres | Segregate | Segregate | Segregate | Isolate | 3 metres | 3 metres |
|  |  | Class 4 | Flammable Solids | Segregate | Refer SDS | Segregate | 3 metres | Okay | 3 metres | Segregate | Segregate | Segregate | 3 metres | Refer SDS |
|  |  |  | Spontaneously Combustible | Segregate | Segregate | Segregate | Segregate | 3 metres | Okay | 3 metres | Segregate | Isolate | 3 metres | 3 metres |
|  |  |  | Dangerous When Wet | Segregate | Refer SDS | Segregate | Segregate | Segregate | 3 metres | Okay | Segregate | Isolate | Refer SDS | Refer SDS |
|  |  | Class 5 | Oxidising Agent | Segregate | Refer SDS | Refer SDS | Segregate | Segregate | Segregate | 3 metres | Refer SDS | Segregate | 3 metres | 3 metres |
|  |  |  | Organic Peroxide | Isolate | Segregate | Segregate | Isolate | Segregate | Isolate | Segregate | Segregate | 0 metre | 3 metres | 3 metres |
|  |  | Class 6 | Toxic and/or Infectious Substances | 3 metres | Refer SDS | 3 metres | 3 metres | 3 metres | Refer SDS | 3 metres | 3 metres | 3 metres | 0 metres | 3 metres |
|  |  | Class 8 | Corrosive | 3 metres | 3 metres | 3 metres | 3 metres | Refer SDS | 3 metres | Refer SDS | 3 metres | 3 metres | Refer SDS | Refer SDS |

| Okay | Dangerous goods of the same class have similar primary hazards and are usually considered compatible. Consult the SDS or supplier. |  | Isolate | Dedicated stores or storage cabinets are recommended. Adequate separation from other buildings and boundaries is required. Consult the SDS or supplier |
| --- | --- | --- | --- | --- |
| 3 metres | These two classes should be kept apart by at least three metres or other suitable control measures. Consult the SDS or supplier. |  | Refer SDS | Segregation of these two classes may be necessary. Consult the SDS or supplier. All Class 9 dangerous goods should be segregated. |
| Segregate | These two classes are likely to interact causing a hazardous event. They must NOT be stored together Consult the SDS or supplier |  | Combustible Liquids | Combustible liquids (including many oils and greases) should be managed as Dangerous Goods Class 3 - Flammable Liquids |